[image: image1.jpg]



PALM SPRINGS AERIAL TRAMWAY TECHNICAL FACTS

This is a double reversible aerial tramway, also known as a Jig-Back Tramway.

The floor of each tramcar makes one to two rotations per trip.

Manufacturer:
Doppelmayr Tramways of Thun, Switzerland

Elevation of lower terminal (Valley Station):
2,643ft / 806m ASL

Elevation of upper terminal (Mountain Station):
8,516ft / 2,596m ASL

Difference of elevation:
5,873ft / 1,791m

Actual length traveled along cables:
12,780ft / 3,895m


2.5 miles

Average gradient:
26 degrees

Maximum:
42 degrees

Number of intermediate towers:
5

The first tower is the tallest at approximately: 
227ft

Car statistics:
Height inside:
8ft 


Diameter:
18ft 


Carriage Height: 25 ft

Weight (carriage, hanger and cabin)

Empty: 


22,000lb / 10,000kg


Maximum Payload:


13,600lb / 6,200kg


Total Weight: 35,600lb / 16,200kg

Capacity of each car:
Approx. 80 passengers plus 


1 car operator

Normal traveling speed:
7.5 meters per second 

or 16 mph (1476 feet per minute)

Maximum existing traveling speed:
10 meters per second  


or approximately 22 mph (1,970 feet per minute) 
Maximum future traveling speed:
11 meters per second 


or approximately 24 mph (2,165 feet per minute)

Maximum traveling speed while passing towers:
8 meters per second 


or approximately 18 mph (approx. 1,575 feet per minute)

System of electric drive:
3 Phase, AC, Regenerative Variable Frequency
Maximum output of AC drive motor:
1,800 HP – 1,343 KW

Continuous output:
1,200 HP – 895 KW

Back-up Engines:



1st:
6.5 meters/second 



12-cylinder diesel engine

2nd: 
2 meters/second



6-cylinder diesel engine

Number and size of cables:


2 x 2 tension to track ropes of 3 inches / 75mm diameter




2 stationary track cables of 1-7/8 inches / 47.5mm diameter


2 stationary track cables of 1-15/16 / 48mm diameter


1 lower hauling rope of 1-9/16 inches / 40mm diameter


1 upper counter rope of 1-3/4 inches / 45mm diameter


1 auxiliary rope of 15/16 inches / 24mm diameter


1 telephone rope of 5/8 inches / 16.2mm diameter, with a fiber optic core


The cable span and number of slack rope carriers

From/To
Cable Span
Number of 



Slack Carriers

Valley Station/Tower 1
1,148ft / 349m
1

Tower 1/Tower 2
3,280ft / 1,000m
5

Tower 2/Tower 3
3,225ft / 983m
4

Tower 3/Tower 4
3,455ft / 1,053m
4

Tower 4/Tower 5
968ft / 295m
1

Tower 5/Mountain Station
695ft / 212m
0
Individual Tower Heights:


Tower 1
227ft / 69.1m


Tower 2
146ft / 44.4m


Tower 3
83ft / 25.3m


Tower 4
65ft / 19.8m


Tower 5
68ft / 20.6m

Each pair of track cables is tensioned by counterweights at the Valley Station, weighing more than 120 tons each.

The haul and counter ropes are tensioned by a counterweight at the Mountain Station weighing 67 tons.

Other rotating trams in the world:
Mt. Titlis

Engelberg, Switzerland


Winter/Summer


Table Mountain


Cape Town, 


South Africa


Year Round

# # #

FOR IMMEDIATE RELEASE





Contact:  Madison Morgan


Public Relations & Events Manager


760-969-4385


Email:  � HYPERLINK "mailto:mmorgan@pstramway.com" �mmorgan@pstramway.com�


Media Site: � HYPERLINK "https://www.pstramway.com/media/" �https://www.pstramway.com/media/�














Page 1 of 3
Page 2 of 3

